Parts

ESP8266

RELAY module

5V adaptor

Wiring details

Low Voltage Side

ESP8266 D1/GPIO5 → Relay IN1

ESP8266 GND      → Relay GND

External 5V +    → Relay VCC

External 5V -    → Relay GND

AC Side

Main Live  → COM

NO         → Bulb Live

Neutral    → Bulb Neutral directly

Modes

| Alexa % | Alexa Value | Mode |
| --- | --- | --- |
| 1–11% | 1–28 | Mode 1 Blink |
| 12–22% | 29–56 | Mode 2 Smooth |
| 23–33% | 57–84 | Red |
| 34–44% | 85–112 | Green |
| 45–55% | 113–140 | Blue |
| 56–66% | 141–168 | Olive |
| 67–77% | 169–196 | Purple |
| 78–88% | 197–224 | Cyan |
| 89–100% | 225–255 | White |

Alexa Commands to Use

Try these: Alexa, set Magic Light to 100 percent

Mode 1 Blink :Alexa, set Magic Light to 10 percent

Mode 2 Smooth: Alexa, set Magic Light to 20 percent

Red: Alexa, set Magic Light to 30 percent

Blue: Alexa, set Magic Light to 40 percent

White: Alexa, set Magic Light to 90 percent

Important Note

When you say: Alexa, turn on Magic Light

Alexa may send: 255

So it will go to Mode 9 / White.

For selecting modes, use: Alexa, set Magic Light to 30 percent

Not just ON/OFF.

How to connect it to Alexa

Use Philips Hue → Discover Devices
✔ Don’t manually create devices
✔ Your code defines what Alexa finds
✔ Each mode appears as a separate device